Functionality of ElzabDr.Dll library

ElzabDr.Dll (Windows version) and elzabdr.a (Linux version) is designed for

- FP-600A,

- ELZAB Omega (1st  and 2nd  generation)

- MERA

fiscal printers.

- Library ElzabDr.dll works in Windows 95/98/ME/XP/2000/VISTA operating systems

- Library elzabdr.a works in Linux. You must pre configure every used serial port by command:

setserial /dev/ttySx uart 16550A

e.g.

setserial /dev/ttyS0 uart 16550A for COM1

setserial /dev/ttyS1 uart 16550A for COM2

In elzabdr.a library there are no functions: sErrMessage, sReceiptItem, sCharsInArticleName, sFillPayment, sFillLines, sReadClock, sDeviceName, sDllVersion, sDllAuthor, sReadUniqueNumber , sReadTotal, sReadSelTotal, sDisplayFP600.

You should use similiar functions: pErrMessage, pReceiptItem, pCharsInArticleName, pFillPayment, pFillLines, pReadClock, pDeviceName, pDllVersion, pDllAuthor, pReadUniqueNumber , pReadTotal, pReadSelTotal, pDisplayFP600.

Library  elzabdr.a does not handle 28800 b/s baudrate.

If there is not specified in function description which type of printer is handled, such function handles each type of fiscal printer.

# I Error handling

Each function returns:

0 – finished successfully

or

non-zero error number.

To see description of error call function pErrMessage.

function pErrMessage(ErrNo : longword;  Message : PChar) longword;

Specification of returned errors:

| ###### Symbolic name | Value | Remarks |
| --- | --- | --- |
| E_OK. | 0 | Finished successfully |
|  | 2 | Impossible to open serial port (no port in operating system or port used by another application) |
| E_SEND_TIMEOUT | 20 | Check connection PC – fiscal printer |
| E_RECV_TIMEOUT | 21 | Check connection PC – fiscal printer and baud rates in printer and computer (baud rate in PC and fiscal printer must be the same) |
| E_IMPROPER_ANS | 23 | Check connection PC – fiscal printer and baud rates in printer and computer (baud rate in PC and fiscal printer must be the same) |
| E_NEGATIVE_ACK | 24 |  |
| E_INIT_FAIL | 99 | Error during initiation of serial port |
| E_PRODUCENT | 100 | Call service |
| E_ZWORA | 101 | Call service (to set service jumper in normal position) or (if MERA printer) reset fiscal printer |
| E_KONTROLER | 102 | Call service |
| E_AWARIA_DR | 103 | Call service |
| E_UZYTKOWNIK | 104 | Set header (user’s data) using service program |
| E_TYLKO_ODCZYT | 105 | Call service |
| E_WYSWIETLACZ | 106 | Check if display is connected (call service) |
| E_CMOS | 107 | Set date and time using service program (if printer in fiscal mode then call service) |
| E_ZALEGLY_R | 108 | Print daily report |
| E_BRAK_PAPIERU | 109 | Change paper (give new roll) and resume printing |
| E_DOK_DO_DRUK | 110 | Wait for finishing printing |
| E_ZN_DO_DRUK | 111 | Wait for finishing printing |
| E_WYDR_ZATRZ | 112 | Resume printing |
| E_STAWKI | 113 | Set tax rates using service program |
| E_PORT | 114 | Open proper serial port (serial port of proper number) |
| E_INSIDE_R | 200 | Impossible during opened receipt |
| E_OUTSIDE_R | 201 | Impossible during closed receipt |
| E_ZNAKI_ZNACZACE | 202 | Change name of article because it has to less significant chars |
| E_STAWKA | 203 |  |
| E_KOMUNIKAT | 204 |  |
| E_ILOSC | 205 |  |
| E_MP | 206 |  |
| E_JEDN | 207 |  |
| E_CENA | 208 |  |
| E_WARTOSC | 209 |  |
| E_UNIEWAZN | 210 | Check the reason of cancelling receipt |
| E_ZERO_PARAG | 211 |  |
| E_DPOZA_ZAKR | 212 |  |
| E_ROZKAZ_NIED | 213 |  |
| E_PPOZA_ZAKR | 214 |  |
| E_ZAKRES | 215 |  |
| E_LINES | 216 |  |
| E_NIESTAWKI | 217 |  |
| ###### E_NIEZEROP | 218 |  |
| E_DATA | 219 |  |
| E_BRAK_RAP | 220 |  |
| E_NUMERY | 221 |  |
| E_MIESIAC | 222 |  |
| E_PARAM | 223 |  |
| E_WERSJA | 224 |  |
| E_RET_PAR_NULL | 225 | Allocate memory for data returned by function |
| E_REZERWA | 226 |  |
| E_IL_CENA | 227 |  |
| E_0PRZESTAW | 228 |  |
| E_60PRZESTAW | 229 |  |
| E_NOPRZESTAW | 230 |  |
| E_CHARACTERS | 231 |  |
| E_XOR | 232 |  |
| E_NONCOMPLETE | 233 |  |
| E_SAME | 234 |  |
| E_NOTPRINTED | 235 |  |
| E_NOOPEN | 236 |  |
| E_BADFORMAT | 237 |  |
| E_EXCEPTION | 999 |  |

Errors which are not described are system errors.

Remark:  The most often reason of E_EXCEPTION error is improper declaration of library function in external application.

## II Data types

Longword – unsigned four bytes number (range 0..4294967295)

PChar  -

when value is transferred into function

PChar – pointer to character chain finished by 0 character; memory is allocated by calling function

When value is transferred from function

PChar – pointer to text data buffer (not shorter than 255 bytes); buffer is filled by called function in ASCIIZ standard.

tBuffer = array [1..256] of byte; this type is used only in RSSequence function,

There are two functions which can help you to create front-office programs:

function SetDebugMode : longword; StdCall;

Switches debug mode on – after calling each function there is displayed message box with parameters transferred into function

function ClearDebugMode : longword; StdCall;

Switches debug mode off. By default debug mode is always switched off.

## III Transmission

Before using communication functions you must set communication parameters.

function CommunicationInit(PortNo : longword; Baudrate : longword; Timeout : longword) : longword;

PortNo – number of serial port from 1 to 255

Baudrate: 9600 b/s or 19200 b/s or 28800 b/s (only for MERA printer)

Timeout – sending to and receiving from fiscal printer timeout. It is recommended to set 10 seconds timeout.

Function opens serial port number PortNo and initiates it with parameters:

- baudrate: as in parameter  Baudrate

- parity: even

- stop bits: 1

- sending timeout:  as in  Timeout parameter,

- receiving timeout: as in  Timeout parameter,

After initialisation it checks type of device connected to serial port, then checks if receipt is open and cancels it.

Function should be called only once before beginning of using fiscal printer from PC-computer program.

Device type can be also identified by DeviceName function.

function CommunicationEnd : longword;

Function closes opened serial port. It should be called only once before closing application.

## III Receipt

function ReceiptConditions : longword;

Checking of all conditions to print receipt. It is not necessary to call it before beginning of printing receipt because ReceiptBegin checks if receipt can be printed.

function ReceiptBegin : longword;

Beginning of receipt. Inside ReceiptBegin calls ReceiptConditions function.

function pReceiptItem(Sale : longword; Name : PChar; TaxRate , Message : longword;Quantity : longword; QC : longword; Unit : PChar; Price : longword; var Value : longword): longword;

Prints receipt item.

Parameters :

Sale  1 – sale item

0 – correction item,

Name: name of receipt item, max 28 characters, longer names are cut, shorter names are completed to 28 chars (by spaces); name must have at least 10 (FP – 600 and Omega 1st  generation) or 5 significant characters (Omega 2nd  generation and MERA).

Significant characters are: small and capital letters, digits, comma, dot.

TaxRate  - tax rate index from 1 to 7 (A=1,B=2, ...G=7)

Depending on printer type:

FP-600 5-tax rates – you can use tax rates from 1 to 4 and 7;

FP-600 7-tax rates, Omega 1st and 2nd  generation and MERA you can use all 7 tax rates

Remark: index 7 of tax rate is always ZWOLNIONA for FP-600 and Omega 1st generation printer. For Omega 2nd  generation printer it is G tax rate (it can be programmed as you wish, but we recommend to define ZWOLNIONA tax rate in this place).

Message: number of additional message in receipt item,

0   (no message)

1 – Z RABATEM

2 – PRZECENA

3 – PROMOCJA

4 – Z KAUCJĄ

5 – BEZ KAUCJI

6 – CENA #1

7 – CENA #2

Quantity 	quantity (to 999999, in Omega 2nd generation till 9999999)

QC 	quantity after comma for Quantity

Unit  	measure unit – max 4 ASCII characters

Price	price in grosze (cents)

Value – if value = 0, then ReceiptItem will internally count Value;

Value is always positive, (for correction item too)

Receipt item value always in grosze;

Remark:

Value:= (Quantity / (10QC)) * Price

Receipt is automatically cancelled if:

- in article name is to less significant characters,

- tax rate is after range,

- maximal message number is out of range,

- maximal quantity, quantity after comma or price are exceeded,

- item value is different from product price multiplied by quantity,

In this case returned error number shows reason of cancelling receipt.

Receipt can be also cancelled because of other reasons, e.g. trying to sell in undefined (reserved) tax rate.

function PackageItem(Param : longword;No :longword ; Quantity : longword; Price : longword): longword;

#### Selling or returning package item:

Param 0 – selling of package item

1 – returning of package item

2 – cancelling selling of package item

3 – cancelling returning of package item

No – package number

Quantity – quantity of package

Price – package price

Position value is calculated automatically; maximum position value is 999999999 gr

function DiscountOrSurcharge(DiscountSurcharge : longword; Amount : longword): longword; StdCall;

#### Discount or surcharge for sale position. Only for MERA printer.

DiscountSurcharge : 1 – amount discount for sale position

0 – amount surcharge for sale position

Amount – amount of discount or surcharge

function pCharsInArticleName(N : PChar) : longword;

Function returns number of significant characters in character chain N.

Name must have at least 10 (or 5 for Omega 2nd generation and MERA) significant characters. Function only informs, because it is checked in pReceiptItem function.

function PrintSubtotal : longword;

Print subtotal on receipt (only on Omega 2nd generation fiscal printer)

function OpenDrawer(DrawerNumber : longword) : longword;

Opens drawer from 1 to 3. For Omega and MERA printer only number 1 make sense, because only one drawer can be connected. For FP-600 drawers 2 or 3 also can be used.

function ReceiptEnd(Discount : longword) : longword;

Discount : discount in percent for all receipt; range 0 - 99

Function closes receipt.

If before calling additional payments (function pFillPayment) and / or additional lines (function pFillLines) and / or currency payments (function pCurrencyPayment1 and pCurrencyPayment2) are set, they will be printed.

Properly finished ReceiptEnd clears variables with additional payments and additional lines.

function ReceiptEndEx(DiscountSurcharge : longword; DiscountSurchargeType : longword) : longword; StdCall;

DiscountSurcharge : percentage or amount display or amount surcharge,

DiscountSurchargeType:

0 – percentage discount; in this case DiscountSurcharge must be from 0 to 99

1 – amount discount; in this case DiscountSurcharge must less than sum of receipt

2 – amount surcharge

Function closes receipt.

Amount discount and surcharge only in MERA printer.

Percentage discount in all types of printers.

If before calling additional payments (function pFillPayment) and / or additional lines (function pFillLines) and / or currency payments (function pCurrencyPayment1 and pCurrencyPayment2) are set, they will be printed.

Properly finished ReceiptEnd clears variables with additional payments and additional lines.

function ReceiptCancel : longword;

Function cancels receipt. Properly finished ReceiptCancel does not clear variables with additional payments and additional lines.

function PrintControl(BeforePrinting : byte) : longword;

BeforePrinting 0 – printout control when printing; function waits for end of printing,

1– before printing; if is not possible to print now, function returns error,

Function controls printout of document (receipt, report etc.). PrintControl is also called internally in all printing functions.

function PrintResume : longword;

Resumes printing interrupted by end of paper.

Printing resume should be called directly after returning by PrintControl E_WYDR_ZATRZ or E_DOK_DO_DRUK.

function pFillPayment(PaymentNo : longword; PaymentName : PChar; Total, Change : longword): longword;

#### Sets additional payment

PaymentNo – payment number  from 1 to 4

PaymentName – payment name – maximum 13 characters

Total – total paid by customer

Change – return money in payment

Function pFillLines(No : longword; Line : PChar; var FreeLines : longword) : longword;

Sets additional lines to print after printout.

No – index of additional line  (from  0 to 20)

Line – content of additional line

FreeLines – quantity of lines which may be used after adding this line.

| Number of line | Beginning of text | N – max characters after beginning |
| --- | --- | --- |
| 0 | ZAPŁACONO | 28 |
| 1 | RESZTA | 31 |
| 2 | SPOSÓB ZAPŁATY: | 22 |
| 3 | ZAPRASZAMY | 27 |
| 4 | OTWARTE | 30 |
| 5 | OBSŁUGUJĄCY | 26 |
| 6 | KOMPUTEROWY | 26 |
| 7 | VOUCHER | 30 |
| 8 | KAUCJA | 31 |
| 9 | ZWROT KAUCJI | 25 |
| 10 | GOTÓWKA | 30 |
| 11 | BEZ NARZUTU | 26 |
| 12 | NUMER | 32 |
| 13 | STAN LICZNIKA | 24 |
| 14 | KLIENT | 31 |
| 15 | KARTA KREDYTOWA | 22 |
| 16 | ILOŚĆ POZYCJI | 24* |
| 17 | (nothing, barcode is printed) | 16* |
| 18 | ZWROTÓW NIE PRZYJMUJEMY | 16* |
| 19 | PUNKTY | 33* |
| 20 | SUMA PUNKTÓW | 27* |

* - only for MERA printers

function ErasePayments : longword;

Clears variables with additional payments.

Always returns 0.

function EraseLines : longword;

Clears variables with additional payments.

Always return 0.

function pCurrencyPayment1(CurrencyName : PChar; Rate : Pchar; Quantity : longword; Subtotal : longword; Send : longword ): longword;StdCall;

Function sends to printer immediately (if Send is equal to 1) or stores to send by ReceiptEnd (if Send is equal to 0) information about foreign currency, its name, exchange rate and subtotal.

Only for MERA printer.

CurrencyName – three characters (symbol of currency); if CurrencyName is EUR it is printed as euro sign

Rate – exchange rate (max 10 characters); leading and trailing spaces are cut

Quantity – quantity of foreign currency (usually 1) connected to exchange rate; for currencies which do not have after comma part (e.g. Ft) you should set 100

Subtotal – to pay in foreign currency

Send – 0 – data of foreign currency are send in end of receipt

1 – data of foreign currency are send immediately

function pCurrencyPayment2(TotalInCurrency : longword; TotalInBaseCurrency : longword; Change : longword ; TypeOfChange : longword; Send : longword ): longword;StdCall;

Function sends to printer immediately (if Send is equal to 1) or stores to send by ReceiptEnd (if Send is equal to 0) information about paid in foreign and base currency, change and type of change.

Only for MERA printer.

TotalInCurrency – paid in foreign currency

TotalInBaseCurrency – paid in base currency

Change – change

TypeOfChange – type of change

0 – change in base currency

1 – change in foreign currency

Send – 0 – data of foreign currency are send in end of receipt

1 – data of foreign currency are send immediately

## IV Reports and other functions

function DailyReport(Unconditionally : longword) : longword;

Prints daily report

Unconditionally – parameter important in Omega 1st and 2nd generation

1 – daily report will be printed always when it can be printed,

0 – daily report will be printed only when something was sold,

(at least one receipt after previous daily report)

function PeriodReport(Fiscal : longword; yy1, mm1, dd1, yy2, mm2, dd2 : longword) : longword;

Prints period report from date to date  - fiscal (detailed) an non-fiscal (shortened, only summary)

### Fiscal

1 – fiscal (detailed) report,

0 – non-fiscal (shortened) report.

yy1,dd1, mm1 – year, month, day of report begin

yy2,dd2, mm2 – year, month, day of report end

function LockedArticlesReport : longword;

Prints locked articles report.

function MonthlyReport(Fiscal : longword; Year,Month : longword) : longword;

Prints monthly report

Fiscal:

1 – fiscal monthly report

0 – non-fiscal (shortened) monthly report.

Year – report’s year (2 digits)

Month – report’s month (2 digits)

function NumberReport(Fiscal : longword; Nr1,Nr2 : longword) : longword;

Prints report by numbers.

### Fiscal:

1 – fiscal report,

0 – non-fiscal report (shortened)

Nr1	- number of first daily report in period number report,

Nr2	- number of last daily report in period number report,

function ReceiptNumber(var Number : longword) : longword;

Reads number of last printed receipt.

Number – receipt number.

Remark : receipt number is cleared after each daily report.

function DailyReportNumber(var Number : longword) : longword;

Reads number of last printed daily report.

Number – number of daily report.

function pReadClock(Time : PChar) : longword;

Reads date and time from fiscal printer.

Time – date, time in format:

yy.mm.dd;hh:mm

function WriteLineFeed(Count : longword) : longword;

Writes number of line feeds after each printout..

Count – quantity of line feeds (from 1 to 30);

Only for FP-600A and Omega printers.

function PrinterStatusReport : longword;

Prints status report.

function ReadVAT (var Count : longword; var A,B,C,D,E,F,G : longword): longword;

Count – number of tax rates read from printer (depends on type of printer):

- in FP600 and FP-600B 5- tax rates it is  4 (5th is always ZWOLNIONA),

- in FP-600 and FP600B 7 tax rates and Omega 1st generation it is 6 tax rates (7th tax rates is always ZWOLNIONA),

- in Omega 2nd generation, Omega F and MERA and newer products it is 7 (all there are free defined, but should be defined as Ministry of Finance described)

A, B, C, D, E, F,G – tax rates in 0,01% or

Value  10000 means tax rate free (ZWOLNIONA), value  25500 means reserved (REZERWA).

Remark: in FP-600 5 – tax rates it is possible set and get tax rates  only in whole percents

To set compatibility with previous versions tax rates are always returned in 0,01%.

function SetVAT (var Count : longword; A,B,C,D,E,F,G : longword): longword;

Count  – returns how many tax rates were written to fiscal printer (as in ReadVat function)

Tax rates should sent in 0,01%. Unused tax rates as 25500, Free (ZWOLNIONA) tax rate as 10000. Function does not control if parameters are properly set. It controls only if tax rates were written to the printer.

Example of setting tax rates in Omega 2nd generation.

A=22,00%, B=7,00%, C=0,00%, D=UNUSED, E=UNUSED, F=UNUSED, G=ZWOLNIONA

Result := SetVat(Count, 2200,700,0,25500,25500,25500,10000);

In FP 600 5 – tax rates, it is possible only writing integer tax rates. Because of this if A is 1250 to the printer will be set 12 % tax rate.

function pDeviceName(Name : PChar) :longword;

Function returns device name recognised by CommunicationInit

Name – device name

Function pDllVersion(Ver : PChar) : longword;

Returns always 0. In Ver variable returns library version.

function pDllAuthor(Auth : PChar) : longword;

Returns always 0. In Auth variable returns copyright information (ELZAB Company property)

function pReadUniqueNumber(Numer : PChar) : longword;

#### Returns in  Number variable unique number of fiscal printer.

function pReadTotal(sTotal : PChar) : longword;

Returns in variable sTotal value of sell in all tax rates since the last daily report. Sale is returned in gr (1 PLN = 100 gr). Calling possible only for 2nd generation Omega and MERA.

If all tax rates are undefined  E_STAWKI error is returned.

function pReadSelTotal(Rate : PChar; sTotal : PChar) : longword;

Returns in variable sTotal value of sales for Rate tax rate (since last daily report). Proper values of Rate: ‘A’, ‘B’, ‘C’, ‘D’, ‘E’, ‘F’, ‘G’, ‘+’ .

Calls:

pReadSelTotal(‘+’, sTotal)  and  pReadTotal(sTotal)

are the same.

Function does not check if tax rate is defined. For undefined (reserved) tax rates function returns E_REZERWA error.

Internal representation of value of sell in Omega and MERA fiscal printer is 5-byte number.

Function only possible for 2nd generation Omega and MERA.

function ChangeTime(Hour : longword; Minute : longword) : longword;StdCall;

Function is used for small corrections of internal printer’s clock.

It is possible only once a day directly after daily fiscal report (when receipt number counter is equal to 0).

Date and time can be corrected in range plus minus 60 minutes.

function pWriteLogo(FileName : PCHar) : longword;StdCall;

Only for Mera printer.

Writes graphical screen saver. Meaning of parameters:

FileName – file name of *.bmp file.

Accepted format:

Monochrome bitmat, not compressed, size  192 x 64 piksels.

function pWriteFooterLines(L1, L2, L3 : PCHar) : longword;StdCall;

Only for Mera printer.

Writes additional lines printed after fiscal logo

L1 – line 1 (max 40 characters),

L2 – line 2 (max 40 characters),

L3 – line 3 (max 40 characters).

function pReadFooterLines(L1, L2, L3 : PCHar) : longword;StdCall;

Only for Mera printer.

Reads additional lines printed after fiscal logo

L1 – line 1

L2 – line 2

L3 – line 3

function NonFiscalPrintoutBegin(Number : longword) : longword;StdCall;

Begins non-fiscal printout (ESC, 73h) of given Number.

function pNonFiscalPrintoutLine(Number : longword; Text : PChar; If0A : longword): longword;StdCall;

Sends one line of non-fiscal printout.

Meaning of parameters

Number – lines number

Text – data of line

If0A – 0 – do not send character 0x0A after Text

1  - send character 0x0A after Text

function NonFiscalPrintoutEnd() : longword;StdCall;

Finishes non-fiscal printout (ESC, 73h).

Causes printout of document sent previously by NonFiscalPrintoutBegin and pNonFiscalPrintoutLine functions.

function DeviceType(var Dev : Device; var Prog : longword) : longword;StdCall;

Returns device type and program version

Dev – device type

Type Device is enumerated type defined as below:

Device = (FP6A5ST, FP6A7ST, OMEGA1, OMEGA2, FP6B5ST, FP6B7ST, OMEGAF, MERA, MERA_F, NAJNOWSZE);

Identifiers have given values

| Identifier | Value |
| --- | --- |
| FP6A5ST | 1 |
| FP6A7ST | 2 |
| OMEGA1 | 3 |
| OMEGA2 | 4 |
| FP6B5ST | 5 |
| FP6B7ST | 6 |
| OMEGAF | 7 |
| MERA | 8 |
| MERA_F | 9 |
| NAJNOWSZE | 10 |

#### Prog – printer’s firmware version

## IV Direct fiscal printer handling

Using programmer manual for fiscal printer and functions below you can yourself program functions not implemented in this library:

function OpenPort(Port:longword; BaudRate:longword;

WriteTimeout,ReadTimeout:longword):longword;

Opens serial port.

Port: port number

BaudRate: 9600 b/s or 19200 b/s or 28800 b/s (only MERA printer)

WriteTimeout: write timeout (set to 5 seconds)

ReadTimeout: read timeout (set to 5 seconds)

Function returns 0 if finished properly.

function RSSequence(ControlCode:longword;  QuantityOfBytesToReceive:longword;               QuantityOfBytessToSend:longword;  var InputBuffer:tBuffer;

OutputBuffer:tBuffer):longword;

Sending escape sequence. Direct handling of fiscal printer. List of escape sequence you can find in programmer manual from ftp.elzab.com.pl.

type tBuffer = array [1..256] of byte

ControlCode – control code (as in programmer manual);

E.g. for reading status byte (ESC, 0x5B) you must send  0x5B  hex (decimal 91).

QuantityOfBytesToReceive – quantity of bytes to receive, including ACK byte

QuantityOfBytesToSend – quantity of bytes to send  (without ESC and ControlCode)

InputBuffer – buffer of bytes from fiscal printer (without ACK answer)

OutputBuffer – buffer of bytes to fiscal printer

Returns 0 (if finished properly) or:

E_SEND_TIMEOUT,

E_RECV_TIMEOUT,

E_IMPROPER_ANS,

E_NEGATIVE_ACK.

Example:

Unique number reading:

No additional data are send to printer, so OutputBuffer array is empty.

Res:=RSSequence(0x71,12,0,InputBuffer, OutputBuffer)

If Res=0 then

Begin

//from inputbuffer[0] to inputbuffer[10] is written unique number ; first byte  06 (ACK)

// is overwritten

End

Else

HandleError(res)

Attention: in Visual Basic you must call function as below:

Dim Res As Long
Dim InputBuffer As String * 256
Dim OutputBuffer As String * 256

InputBuffer = String(255, 0)
OutputBuffer = String(255, 0)
Mid(tablOutput, 1, 5) = String(5, Chr(&H30))

Res = RSSequence(&HFD, 1, 5, InputBuffer, OutputBuffer)

Function ClosePort : longword;

Closing serial port.

Properly finished function returns 0, non - zero values are errors.

## VI Functions for programming languages without references/pointers

In case of programming language which does not allow to use references/pointers pleas use functions listed below and LastError function to check if error occured.

function  LastError : longword; StdCall; //returns and clears last error

function xReceiptNumber : longword; StdCall; //(equivalent of  ReceiptNumber)

function xDailyReportNumber : longword; StdCall; //( equivalent of DailyReportNumber)

function xIsCodePage1250: longword; StdCall; //( equivalent of IsCodePage1250)

function xDeviceType: Device; StdCall; //( equivalent of DeviceType parameter Device)

function xProgramVersion: longword; StdCall; //( equivalent of DeviceType parameter Prog)

type vat =

Ile, A, B, C, D, E, F, G : longword;

end;

function xReadVAT: vat; StdCall; //( equivalent of ReadVAT)

function xFillLines(Nr : longword, Line : PChar) : longword; StdCall; //(odpowiednik pFillLines)

function xDllVersion : PChar; StdCall; //( equivalent of pDllVersion)

function xDllAuthor : PChar; StdCall; //( equivalent of pDllAuthor)

function xErrMessage(ErrNo : longword) : PChar; StdCall; //( equivalent of pErrMessage)

function xDeviceName : PChar; StdCall; //( equivalent of pDeviceName)

function xReadStatus(StatusNo : longword) : longword; StdCall; //(odpowiednik ReadStatus)

function xReadClock : PChar; StdCall; //(odpowiednik pReadClock)

function xReadUniqueNumber : PChar; StdCall; //(odpowiednik pReadUniqueNumber)

function xReadTotal : PChar; StdCall; //(odpowiednik pReadTotal)

function xReadSelTotal(Rate : PChar) : PChar; StdCall; //(odpowiednik pReadSelTotal)

----------------------------------------------------------------------------------------------------------------------------------------

If case of errors and remarks please mailto:  markefka@elzab.com.pl

or

tk@elzab.com.pl