Communication protocol for LAN Terminal

# General characteristics

- Each Terminal has its own IP address. This number is stored in Terminal’s memory and can be configured over LAN. Each Terminal’s IP must be unique in given local network.

To communicate with Terminal you must know its IP – address.

To communicate with Terminal you must use UDP protocol (port number 1001).

Reception of each frame with data must be confirmed by reversible frame with data or acknowledgement.

If acknowledgement did not reach in 1 second, the frame must be send again (at most 3 times).

- Used control characters:

| Char | Value | ## Description |
| --- | --- | --- |
| STX | 02H | Beginning of the text block |
| ETX | 03H | End of the text block |
| ACK | 06H | Acknowledgement of receiving frame with data |
| LF | 0AH | Changing line |
| CR | 0DH | End of data |

- Data frame

| STX | <FRAME_NO> | <FRAME_DATA> | ETX |
| --- | --- | --- | --- |
| 1 | 1 | Variable length (1..1024) | 1 |

- Frame with acknowledgement

| STX | <FRAME_NO> | ACK | ETX |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |

- <FRAME_NO>

ASCII character from 0x80 to 0xFF (128..255)

- <FRAME_DATA>

ASCII characters from 0x20 to 0xFF (32..255) and LF and  CR

<FRAME_DATA> = <SEQUENCE><DATA>

or

<FRAME_DATA> = <SEQUENCE>

<SEQUENCE> – sequence byte, ASCII character from 0x20 to 0xFF (32..255)

<DATA> –ASCII characters from  0x20 to 0xFF (32..255) and  LF and CR, variable length depending on type of sequence

# Syntax of sequence PC-computer –Terminal

## Sending barcode from Terminal to PC-computer

| TERMINAL | PC-COMPUTER |
| --- | --- |
| STX FRAME_NO 1 BARCODE ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During process of checking barcode Terminal is the device of MASTER type and PC-computer is the device of SLAVE type.

FRAME_NO generates Terminal and sends it in the frame with barcode data.

PC-computer after receiving answer confirms the frame by sending frame with ACK and FRAME_NO.

BARCODE – ASCII characters of barcode, finished by CR character

## Sending key code from Terminal to PC-computer

| TERMINAL | PC-COMPUTER |
| --- | --- |
| STX FRAME_NO 2 KEY ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During process of checking key Terminal is the device of MASTER type and PC-computer is the device of SLAVE type.

FRAME_NO generates Terminal and sends it in the frame with barcode data.

PC-computer after receiving answer confirms the frame by sending frame with ACK and FRAME_NO.

KEY – one  ASCII character in range  ‘0’...’9’, ‘A’...’F’ or special code ‘=’  (after pressing together most left and most right key – testing communication with PC)

## Displaying text with clearing previous contents

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 4 CHARS_DISP SIGNAL ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving answer confirms the frame by sending frame with ACK and FRAME_NO.

CHARS_DISP – characters to display (maximum 40 characters, if there is less characters to display, line must be finished by  LF character)

SIGNAL – number of sound signal:

| 1 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Possible values: from 0 to 127

Signal is generated from the LSB b0

One bit lasts 71 ms. Bit equal to 1 means signal, bit equal to 0 means no signal.

## Displaying text without clearing previous contents

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 5 START CURSOR CHARS_DISP SIGNAL ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving answer confirms the frame by sending frame with ACK and FRAME_NO.

START – place on display where the text starts; two ASCII characters from ‘01’ to ‘40’, e.g. ‘35’ means 15th character in second line of display

CHARS_DISP – characters to display (maximum 40 characters, if there is less characters to display, line must be finished by  LF character)

CURSOR – place on display where cursor blinks; two ASCII characters from ‘00’ to ‘40’, e.g.  ‘35’ means 15th character in second line of display , ‘00’ means no blinking cursor

SIGNAL – number of sound signal:

| 1 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Possible values: from 0 to 127

Signal is generated from the LSB b0

One bit lasts 71 ms. Bit equal to 1 means signal, bit equal to 0 means no signal.

## Generating sound signal

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 6 SIGNAL ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving answer confirms the frame by sending frame with ACK and FRAME_NO.

SIGNAL – number of sound signal:

| 1 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Possible values: from 0 to 127

Signal is generated from the LSB b0

One bit lasts 71 ms. Bit equal to 1 means signal, bit equal to 0 means no signal.

## Reading display contents

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 7 ETX |  |
|  | STX FRAME_NO 7CHARS_DISP CURSOR ETX |
| STX, FRAME_NO, ACK, ETX |  |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving question confirms the frame by sending frame with ACK, FRAME_NO and data.

PC-Computer after receiving answer confirms the frame by sending frame with ACK, FRAME_NO.

CHARS_DISP – characters to display (maximum 40 characters, if there is less characters to display, line must be finished by  LF character)

CURSOR – place on display where cursor blinks; two ASCII characters from ‘00’ to ‘40’, e.g. ‘35’ means 15th character in second line of display , ‘00’ means no blinking cursor

## Reading status of terminal

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 8 ETX |  |
|  | STX FRAME_NO 8 DISP STATUS VERSION ETX |
| STX, FRAME_NO, ACK, ETX |  |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving question confirms the frame by sending frame with ACK, FRAME_NO and data.

PC-Computer after receiving answer confirms the frame by sending frame with ACK, FRAME_NO.

DISP –type of display in terminal (one ASCII character):

always 1 – LCD 2x20 characters

STATUS – terminal status:

| 1 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

b0 = 1 – displaying advertisements ON

b1 = 1 – handling barcode scanner ON

b2 = 1 – barcode scanner power supply ON

b3 = 1 – backlight ON

b4 = 1 – display out of order

b5 = 1 – low CMOS memory voltage

b6 = reserved

VERSION – terminal’s program version (max 20 characters, text must be finished by CR character)

## Programming advertisements

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO 4 NO TIME ATTR TEXT ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence TERMINAL=SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

NO – advertisement’s number, range from 0 to 9

TIME – time of displaying in seconds, two ASCII characters, e.g. ‘03’ means 3 seconds ,

if TIME = ‘00’ then text is displayed till scanning of the barcode

ATTR – way of displaying advertisement:

| 1 | B6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

b0 – b5 – number of displaying way:

0 – text displayed without any additional effects,

1 – text put in from right side of display without deleting previous display,

2 – text put in from left side of display with deleting previous display,

3 – text put in from right side of display with deleting previous display,

4 – text put in from left side of display without deleting previous display,

b6 – text is continuously put in,

b7 – always 1

TEXT – advertisement’s text: 2 lines by 20 characters (maximal 40 chars, if less signs to display in the line, they must be finished by LF character)

## Programming fixed texts

| PC-Computer | Terminal |
| --- | --- |
| STX FRAME_NO 5 NO TIME TEXT ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal =SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

NO – number of fixed text, range from 0 to 6

TIME – time of displaying in seconds, two ASCII characters, e.g. ‘03’ means 3 seconds ,

if TIME = ‘00’ then text is displayed till scanning of the barcode

TEXT – fixed text, 1 line 20 chars (maximal 20 chars, if less signs to display in the line, they must be finished by LF character)

Default fixed texts:

| NUMBER | TEXT |
| --- | --- |
| 0 | TERMINAL LAN |
| 1 | WERSJA PROGR. SPR4.02 |
| 2 | BRAK KOMUNIKACJI |
| 3 | Z SERWEREM |

## Enabling displaying advertisements

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO B ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal =SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

## Enabling/disabling handling of barcode reader

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO C STATUS ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal = SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

STATUS:

0 – handling of barcode reader disabled

1 – handling of barcode reader enabled

## Switching on/off power supply of barcode reader

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO D STATUS ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal =SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

STATUS:

0 – power supply of barcode reader disabled

1 – power supply of barcode reader enabled

## Switching on/off backlight

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO E STATUS ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal =SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

STATUS:

0 – backlight off

1 – backlight on

## Writing ASCII strings to terminal’s memory

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO F BANK STRING ETX |  |
|  | STX, FRAME_NO, ACK, ETX |

During this sequence Terminal =SLAVE, PC-COMPUTER=MASTER

FRAME_NO is generated by PC-computer and insert into frame with asking.

In the frame with the answer Terminal sends received from PC-Computer FRAME_NO. Owing to this, this frame is also acknowledgement.

BANK – number of memory’s bank, two  ASCII characters in range from ‘00’ to ‘99’

STRING  – always 128 ASCII characters

## Reading ASCII strings from terminal’s memory

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO G BANK ETX |  |
|  | STX FRAME_NO G STRING ETX |
| STX, FRAME_NO, ACK, ETX |  |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving question confirms the frame by sending frame with ACK, FRAME_NO and data.

PC-Computer after receiving answer confirms the frame by sending frame with ACK, FRAME_NO.

BANK – number of memory’s bank, two  ASCII characters in range from ‘00’ to ‘99’

STRING  – always 128 ASCII characters

## Reading code of recently pressed key

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO H ETX |  |
|  | STX FRAME_NO H KEY ETX |
| STX, FRAME_NO, ACK, ETX |  |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving question confirms the frame by sending frame with ACK, FRAME_NO and data.

PC-Computer after receiving answer confirms the frame by sending frame with ACK, FRAME_NO.

KEY – one ASCII character in range ‘0’...’9’, ‘A’...’F’ (or space if no key was read)

## Reading code of recently scanned barcode

| PC-COMPUTER | TERMINAL |
| --- | --- |
| STX FRAME_NO I ETX |  |
|  | STX FRAME_NO I BARCODE ETX |
| STX, FRAME_NO, ACK, ETX |  |

During process of checking key Terminal is the device of SLAVE type and PC-computer is the device of MASTER type.

FRAME_NO generates PC-Computer and sends it in the frame to the Terminal.

Terminal after receiving question confirms the frame by sending frame with ACK, FRAME_NO and data.

PC-Computer after receiving answer confirms the frame by sending frame with ACK, FRAME_NO.

BARCODE – ASCII characters of barcode, finished by CR character

The newest version of library is available always on ftp.elzab.com.pl

Any comments can be send to tk@elzab.com.pl